dl32_1 (1x1000) rpm

dl32_2 (g) (side)

dl32_3 (mph)

dl32_4 (PSIg)

dl32_5 (g) (accel)

signal input is from Digital Speed Sensor, 5 volts -  24 pulses per revolution

Harness 29I red/white wire

(not ecu pin 29  5 volts - two pulses per revolution)

The range to 140 mph seems to be accurate.  I don’t think the voltage ever gets to 5 volts .

PSI is from Autometer 4777 Boost gauge

PSIg	Volt

-14.7	0.000

10.0	2.000

15.0	2.410

29.4	4.600

HP-z31:

?Vehicle weight in lb = m[3410]

?i Name of RPM channel = rpm [DL32_1]

?i Name of Acceleration channel (g) = ACCEL {g}[DL32_5]

?i Name of speed channel (mph) = mph{mph}[DL32_3]

?Frontal area of vehicle (sqft) = Ar[20.08]

?Drag coefficient = Cd[0.38]

?Current atmospheric pressure (inHg) = atmp[29.92]

?Current temperature (degF) = temp[62]

v = mph * 1.4672645086030913

#SAE correction factor

SAEcf = (1.18 * ((29.235/atmp) * sqrt((temp + 459.67)/536.67)) )- 0.18

#air density

density = 0.0412236 * atmp/ (temp + 459.67)

# tire radius in ft

#R = td/24

# aerodynamic draq in lbforce

Fa = 0.5 * density * (v^2) * Ar * Cd

F = m * ACCEL + Fa

powr  =v * F * SAEcf/550

MC(RWHP;hp;0) = powr

# torque from this:

MC(RWtorque;lbft;0) = powr * 5252/rpm

Torque-z31:

?Vehicle weight in lb = m[3410]

?i Name of RPM channel = rpm [DL32_1]

?i Name of Acceleration channel = ACCEL {g}[DL32_5]

?i Name of speed channel = mph{mph}[DL32_3]

?Frontal area of vehicle (sqft) = Ar[20.08]

?Drag coefficient = Cd[0.38]

?Current atmospheric pressure (inHg) = atmp[29.92]

?Current temperature (degF) = temp[70]

# convert MPH to Ft/Sec (mult by magic number):

v = mph * 1.4672645086030913

#SAE correction factor

SAEcf = (1.18 * ((29.235/atmp) * sqrt((temp + 459.67)/536.67)) )- 0.18

#air density

density = 0.0412236 * atmp/ (temp + 459.67)

# tire radius in ft

#R = td/24

# aerodynamic draq in lbforce

Fa = 0.5 * density * (v^2) * Ar * Cd

F = m * ACCEL + Fa

powr  =v * F * SAEcf/550

MC(RWtorque;lbft;0) = powr * 5252/rpm